MILLENNIUM LOCKS CORP.

# 4898 Lalande Blvd, Pierrefonds, Quebec, Canada, H8Y 1V7 Tel: +1(514) 824-5625  Fax: +1(514) 944-5625

ULIX ELECTRO-MECHANICAL LOCK

PROGRAMMER’S INSTRUCTION

PRODUCT OVERVIEW

The lock is operated via a 10 digit numeric keypad plus a ‘Prog’ and ‘End’ key. There are 3 Light Emitting Diodes ( L.E.D.’s) located at the top of the key pad which provide information as to the locks status. Once a valid code 6 digit has been entered, or two 6 digit codes, if the lock is set in dual control mode, will allow the bolt can be withdrawn by turning the knob, or the optional extra 'key operated cylinder knob' located in the centre of the key pad.

***NEW *** The programming code is now reseting user1 and user2 to the factory default of 111111 and 222222. Factory default programming code is 000000 and this code will not operate the lock. User must change that factory programming code to anything else before being able to use the lock. It is very important to use a programming code very difficult to guess.

# LOCK MODELS

The lock is available in four different bolt models:

ULIX-1:  Manual retraction and automatic extension of the lock bolt (after a short delay)

ULIX-2 : Manual retraction and extension of the lock bolt.

ULIX-3 : Manual retraction of the deadlocking slam lock bolt.

ULIX-4 : Manual retraction of the non-deadlocking slam lock bolt.

The bolt retraction knob can be substituted by a Medeco cylinder.

###### FUNCTIONS

All lock models have the following functions available:

Single user mode

Dual control mode

Time delay for either single or dual control mode

Up to 99 minutes delay and 9 minutes open window.

CHANGE FACTORY SET PROGRAMMING CODE  0,0,0,0,0,0

To change the Programming code, press the ‘Prog’ key and enter 0,0,0,0,0,0. The RED L.E.D.s. will light, enter a new 6-digit code The GREEN L.E.D. will light. Re-enter the same 6-digit code to confirm, and if correct the L.E.D. will shut off.

‘Prog’ ~ ‘000000’ ( RED L.E.D. will light up)

‘new code’ ( GREEN L.E.D. will light up)

‘new code confirmation’  (GREEN L.E.D. will shut off).

NOTE - If an error is made during the code changing sequence the RED & GREEN L.E.D. will flash and the buzzer will beep to indicate an error, at which time the lock will revert back to the original code.

If at any time a code is not entered correctly the sounder will beep 4 times. At this stage it is necessary to start the procedure again. After 3 incorrect entry attempts, the lock will shut down for 10 minutes and the red and yellow LED’s will flash alternately at 2 second intervals.

SETTING A TIME DELAY AND OPEN WINDOW VALUE

In order to configure the lock for time delay, the programming code must be entered, at which time the red LED will light. The lock type prefix number (see chart below)  must be entered followed by the time delay value and then the open window value.

The time delay value must be entered using  only ‘2 digits’ . For zero time delay press '00', and subsequent values up to ‘99’ for 99 minutes delay.

For the open window value, only ‘1 digit’ must be pressed ‘0’ = 30 seconds which is the minimum open window allowed, up to a maximum of 9 minutes.

| Prefix | Mode | Lock model |
| --- | --- | --- |
| 0 | Independent Users | ULIX-1, ULIX-2, ULIX-4 |
| 1 | Independent Users | ULIX-3 |
| 2 | Dual Control | ULIX-1, ULIX-2, ULIX-4 |
| 3 | Dual Control | ULIX-3 |

Example :- For a single user lock with a square bolt and a programming code of 383838 who requires a 10 minute delay and a 2 minute open window the setting would be made as follows.

Enter 6 digit programming code 383838

Enter lock prefix '0' followed by '10' followed by '2'.

So, “0102”

After the last digit has been pressed of the new 4 digit lock prefix and time delay value code has been entered, the RED ~ GREEN ~ YELLOW will then flash in sequence and the keypad will beep to confirm acceptance of the new instruction.

NOTE

If the lock has an existing time delay programmed in it,  then the action of entering the programming code will start the time delay.

The RED L.E.D. will flash every second during the time delay period. At the end of the time delay period the GREEN L.E.D. will flash every second during the open window period.

During the open window period re-enter the programme code, followed by the 4-digit program (lock prefix number and the new time delay and open window value).

After the last digit has been pressed of the new 4 digit lock prefix and time delay value code has been entered, the RED ~ GREEN ~ YELLOW will then flash in sequence and the keypad will beep to confirm acceptance of the new instruction.

PROGRAMMING CODES

| Lock Model | User Mode | Delay | Open Window | Programming |
| --- | --- | --- | --- | --- |
| ULIX-1, ULIX-2, ULIX-4 | Independent | 00 | 0 = 30 sec | ‘0000’ |
| ULIX-1, ULIX-2, ULIX-4 | Independent | 10 | 2 | ‘0102’ |
| ULIX-1, ULIX-2, ULIX-4 | Dual Control | 05 | 1 | ‘2051’ |
| ULIX-1, ULIX-2, ULIX-4 | Dual Control | 38 | 7 | ‘2387’ |
| ULIX-3 | Independent | 00 | 0 | ‘1000’ |
| ULIX-3 | Independent | 03 | 1 | ‘1031’ |
| ULIX-3 | Dual Control | 20 | 4 | ‘3204’ |
| ULIX-3 | Dual Control | 33 | 3 | ‘3333’ |