MILLENNIUM LOCKS CORP.

# 4898 Lalande Blvd, Pierrefonds, Quebec, Canada, H8Y 1V7 Tel: +1(514) 824-5625  Fax: +1(514) 944-5625

ULIX ELECTRO-MECHANICAL LOCK

USER’S INFORMATION

PRODUCT OVERVIEW

The lock is operated via a 10 digit numeric keypad plus a ‘Prog’ and ‘End’ key. There are 3 Light Emitting Diodes ( L.E.D.’s) located at the top of the key pad which provide information as to the locks status. Once a valid code 6 digit has been entered, or two 6 digit codes, if the lock is set in dual control mode, will allow the bolt can be withdrawn by turning the knob, or the optional extra 'key operated cylinder knob' located in the centre of the key pad.

# LOCK MODELS

The lock is available in four different bolt models:

ULIX-1:  Manual retraction and automatic extension of the lock bolt (after a short delay)

ULIX-2 : Manual retraction and extension of the lock bolt.

ULIX-3 : Manual retraction of the deadlocking slam lock bolt.

ULIX-4 : Manual retraction of the non-deadlocking slam lock bolt.

The bolt retraction knob can be substituted by a Medeco cylinder.

###### FUNCTIONS

All lock models have the following functions available:

Single user mode

Dual control mode

Time delay for either single or dual control mode

Up to 99 minutes delay and 9 minutes open window.

SINGLE USER ACCESS CODES

The ULIX lock is supplied with two sets of 6 digit access codes

Each 6 digit code will open the lock and allows the user/s to change their own codes.

DUAL USER MODE

The ULIX lock is supplied with two sets of 6 digit access codes

Both 6 digit codes are required to open the lock and allows the users to change their codes.

TIME DELAY - SINGLE USER

If the lock has been supplied with this option, entering either of your two 6 digit codes will start the time delay function. At the end of the time delay period the green L.E.D. will light and you will have to re-enter the code that started the time delay function, within the open window period, in order to open the lock.

TIME DELAY - DUAL USER

If the lock has been ordered with this option, entering one of the 6 digit codes will start the time delay function. At the end of the time delay period the second code has to be entered within the open window period, in order to open the lock.

LOW BATTERY WARNING

When the lock senses that the batteries are nearing the end of their useful life, this will be indicated by the Yellow L.E.D. flashing. If this warning are ignored then the lock will function for a further 32 cycles after which the lock will fail, in the locked position.  If at this time any button on the keypad is pressed, the Yellow L.E.D. will flash and the sounder will bleep for 30 seconds. Even though the batteries have insufficient power to operate the lock and they are removed to be replaced with new batteries the codes are not lost as they are retained in the locks memory.

INCORRECT CODE ENTRY LOCKOUT

If three incorrect code entries are made, the lock will shut down for 10 minutes, and further code entries will be rejected during this time. During this shutdown time the Red and Yellow L.E.D.s will flash alternately every 2 seconds.

OPERATING THE LOCK FOR THE FIRST TIME

WITH THE FACTORY SET CODES

The lock is supplied with two sets of 6 digit codes

To ensure complete security both of these must be changed to new numbers. Either of the two codes will open the lock except when the lock is set in dual control, and then both codes will be required.

Factory set 1st code is 1,1,1,1,1,1,

Factory set 2nd code is 2,2,2,2,2,2,

USER TO CHANGE ITS OWN CODE

This operation should be carried out with the safe door in the open position.

If the lock has a time delay function enabled then you should wait for the time delay to run and then open the safe door.

‘Prog button’ ~ ‘existing code 6 digit code’ ~ (RED L.E.D. lights) ~ ‘new 6 digit code’

(GREEN L.E.D. will light) ~ ‘re-enter new 6 digit code’ ~ (GREEN L.E.D. will shut off).

Test lock 3 times with the safe door in the open position.

NOTE

If the code change sequence is not completed within 10 seconds the RED AND GREEN L.E.D. will flash and the sounder will bleep at which time the lock will cancel the operation and revert back to the original code that was being used.

If an error is made during the code changing sequence the RED & GREEN L.E.D. will flash and the sounder will bleep to indicate an error, at which time the lock will revert back to the original code.

The 'END' button can be pressed at any time to clear an incorrect partial entry.

SINGLE  USER - WITHOUT TIME DELAY

When each digit of either code is entered, the red LED will light up and the key pad will bleep. If the correct code has been entered, the green LED will flash after the last digit has been pressed. The lock can now be opened by turning the knob in the centre of the keypad. This must be done within 5 seconds or the lock will re-lock itself automatically.

If an incorrect code has been entered then the RED and Green LED's will flash alternately and the keypad will bleep.

SINGLE USER - WITH TIME DELAY

When each digit of either code is entered, the red LED will flash and the key pad will bleep. If a correct code has been entered, the green LED will flash  after the last digit has been pressed.

The RED L.E.D. will then flash every second of the time delay period. At the end of the time delay period the GREEN L.E.D. will flash for every second of the open window time.

The code that started the time delay must be re-entered at this time.

When each digit of the code is entered, the red LED will flash and the key pad will bleep. If a correct code has been entered, the green LED will flash after the last digit has been pressed and then the lock can be opened by turning the knob in the centre of the keypad.

If a code is not entered within the open window time the lock will relock, itself and the opening sequence will have to be started again

If an incorrect code has been entered then the RED and Green LED's will flash alternately and the keypad will bleep.

DUAL USER - WITHOUT TIME DELAY

When each digit of the two codes is pressed the red L.E.D. will flash and the key pad will bleep. When the first correct code has been entered, the green L.E.D. will flash and the keypad will bleep, the second code should be entered at this time. When each digit of the second code is pressed the red L.E.D. will flash. When the last correct digit has been entered of the second code the green L.E.D. will flash once. The lock bolt can now be retracted by turning the knob in the centre of the keypad. This must be done within 5 seconds or the lock will re-lock itself automatically.

If an incorrect code has been entered then the RED and Green LED's will flash alternately and the keypad will bleep.

DUAL USER - WITH TIME DELAY

Only one code is required to start the time delay. When each digit of either code is entered, the red LED will flash and the keypad will bleep. If a correct code has been entered, the green LED will flash after the last digit has been pressed.

The RED L.E.D. will then flash every second of the time delay period. At the end of the time delay period the GREEN L.E.D. will flash for every second of the open window time.

The second code must be entered at this time.

When each digit of the code is entered, the red LED will flash and the

keypad will bleep. If a correct code has been entered, the green LED will flash after the last digit has been pressed and then the lock can be opened by turning the knob in the centre of the keypad.

If a code is not entered within the open window time the lock will relock, itself and the opening sequence will have to be started again

If an incorrect code has been entered then the RED and Green LED's will flash alternately and the keypad will bleep.